2. Μεταγωγέας (switch) αυξημένων δυνατοτήτων

| Τεχνικά Χαρακτηριστικά | Απαίτηση | Απάντηση προμηθευτή |
| --- | --- | --- |
| Τύπος | Μεταγωγέας (switch) |  |
| Αριθμός τεμαχίων | 5 τεμ. |  |
| 24 × 10/100/1000 Base-T Ethernet ports και 4 x 1/10/25GE SFP28 και 2 x 40/100GE QSFP28 ports | NAI |  |
| One extended slot | ΝΑΙ |  |
| 1+1 power backup | NAI |  |
| Forwarding performance: 450 Mpps τουλάχιστον | NAI |  |
| Switching capacity: 648 Gbps τουλάχιστον | NAI |  |
| FAN: |  |  |
| Number of fans included | 2 |  |
| FAN Maximum power consumption: 21.6 W | NAI |  |
| FAN Maximum fan speed: 24500±10% revolutions per minute (RPM) | NAI |  |
| FAN Maximum wind rate: 31 cubic feet per minute (CFM) | NAI |  |
| FAN Hot swap: Supported | NAI |  |
| Switch must support an intelligent upgrade feature. Specifically, switches must obtain the version upgrade path and download the newest version for upgrade from an online upgrade platform. | NAI για 60 Months |  |
| Preloading the version should be supported, which greatly shortens the upgrade time and service interruption time | NAI |  |
| User management | Unified user management 802.1X authentication MAC authentication Traffic- and duration-based accounting User authorization based on user groups, domains, and time ranges |  |
| MAC | Automatic MAC address learning and aging 256K MAC entries (MAX) Static, dynamic, and blackhole MAC address entries Source MAC address filtering MAC address learning limiting based on ports and VLANs |  |
| VLAN | 4K VLANs simultaneously Access mode, Trunk mode and Hybrid mode Default VLAN Private VLAN QinQ and enhanced selective QinQ VLAN Stacking Dynamic VLAN assignment based on MAC addresses |  |
| ARP | ARP Snooping |  |
| IP routing | IPv4 dynamic routing protocols such as RIP v1/v2, OSPF v1/v2, IS-IS, and BGP 256K IPv4 Routes IPv6 dynamic routing protocols such as RIPng, OSPFv3, ISISv6, and BGP4+ 100K IPv6 Routes Routing Policy, Policy-Based Routing, ECMP |  |
| Segment Routing | SRv6 BE (L3 EVPN) BGP EVPN SRv6 configuration through NETCONF |  |
| Multicast | IGMPv1/v2/v3 and IGMP v1/v2/v3 Snooping PIM-DM, PIM-SM, and PIM-SSM Fast-leave mechanism Multicast traffic control Multicast querier Multicast protocol packet suppression Multicast VLAN |  |
| MPLS | MPLS-LDP MPLS-L3VPN MPLS Qos |  |
| QoS | Traffic classification based on Layer 2 headers, Layer 3 protocols, Layer 4 protocols, and 802.1p priority Actions such as ACL, Committed Access Rate (CAR), re-marking, and scheduling Queuing algorithms, such as PQ, DRR, and PQ+DRR Congestion avoidance mechanisms such as WRED and tail drop Traffic shaping 8 queues on each interface Network Slicing（VLAN/VxLAN/SRv6） |  |
| Ethernet loop protection | STP (IEEE 802.1d), RSTP (IEEE 802.1w), and MSTP (IEEE 802.1s) VLAN-based Spanning Tree (VBST) BPDU protection, root protection, and loop protection G.8032 Ethernet Ring Protection Switching (ERPS) |  |
| Reliability | M-LAG Service interface-based stacking Maximum number of stacked devices Stack bandwidth (Bidirectional) Link Aggregation Control Protocol (LACP) Virtual Router Redundancy Protocol (VRRP) and Bidirectional Forwarding Detection (BFD) for VRRP BFD for BGP/IS-IS/OSPF/static routes Eth-OAM 802.1ag(CFM) Smartlink LLDP, LLDP-MED |  |
| System management | Console terminal service Telnet/IPv6 Telnet terminal service SSH v1.5 SSH v2.0 SNMP v1/v2c/v3 FTP, TFTP, SFTP BootROM upgrade and remote in-service upgrade Hot patch User operation logs Open Programmability System (OPS) Streaming Telemetry Registration Center Deployment |  |
| Security and management | NAC Port-security RADIUS and HWTACACS authentication for login users Management by Command Line Interface(CLI) Command line authority control based on user levels, preventing unauthorized users from using command configurations Defense against DoS attacks, Transmission Control Protocol (TCP) SYN Flood attacks, User Datagram Protocol (UDP) Flood attacks, broadcast storms, and heavy traffic attacks IPv6 RA Guard CPU hardware queues to implement hierarchical scheduling and protection for protocol packets on the control plane Remote Network Monitoring (RMON) Secure boot Macsec(IEEE 802.1ae) Deception Port mirroring Dynamic ARP Inspection IP Source Guard |  |
| Hardware Specifications |  |  |
| CPU | 2 GHz |  |
| Cores | 4 |  |
| Storage | 4 GB |  |
| Flash memory | 2 GB |  |
| Device buffer | 12 MB |  |
| Power | 2x 180 W AC (pluggable) |  |
| Reliability MTBF (year) | Τουλάχιστον 95.25 |  |
| Warranty | 5 χρόνια |  |